Instructions:

- Use this form to complete a self-inspection of your lab to ensure compliance with lab safety requirements. Lab self-inspection is recommended on a monthly basis, required at a minimum on a semi-annual basis.

- Print the form and complete the inspection by walking through the lab and observing lab activities. For all items marked “No”, develop and implement a corrective action plan. Save the inspection with other lab records.

- Notes:

  - This form is electronically fillable.

  - CTI stands for corrected at time of inspection

  - N/A stands for not applicable.

| Date of Inspection: |  | Conducted By: |  |
| --- | --- | --- | --- |
| Building: |  | Room Number(s): |  |
| Principal Investigator: |  | Department: |  |

| # | Item | Yes | No | CTI | N/A | For all items marked “No”, write corrective action plan: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Corrective Action | Person Responsible | Due Date |
| Administrative Controls |  |  |  |  |  |  |  |  |
| Documentation/Training |  |  |  |  |  |  |  |  |
|  | Lab has knowledge of the EHS web page to access all necessary lab safety-related documents (policies, forms, templates, etc.) NOTE: it is recommended that the page be bookmarked by lab members. www.ehs.gatech.edu |  |  |  |  |  |  |  |
|  | Training documentation is present in the lab or other accessible location: - Required: Lab Safety 101 (every 3 years), Right-to-Know (annual) - Process-specific: General Biosafety (every 3 years), Bloodborne Pathogens (annual), Recombinant DNA (every 3 years), Shipment of Dangerous Goods (every 2 years), Using Chemical Inventory System-EHSA (one time), and Fire Safety (one time), Receipt of Hazardous Materials (one time) or others as appropriate. NOTE: Use the EHS Training Tool to determine what trainings are applicable and find links to access or registering for specific classes. |  |  |  |  |  |  |  |
|  | Lab has an up-to-date biosafety approval(s): - Biological Materials Safeguards Committee for work with biological/infectious agents or biological toxins and/or; - Institutional Biosafety Committee for research involving recombinant or synthetic nucleic acid molecules). |  |  |  |  |  |  |  |
|  | Lab maintains an inventory log for Select Agent Toxins in Exempt Quantities and/or DEA Controlled Substances. |  |  |  |  |  |  |  |
|  | Lab uses EHS personnel to ship dangerous goods for them. Dangerous goods include but are not limited to hazardous chemicals, radioactive material, infectious/potentially infectious materials, dry ice, and genetically modified organisms/microorganisms. |  |  |  |  |  |  |  |
| Signage/Lab Postings |  |  |  |  |  |  |  |  |
|  | Doors leading into the lab are labeled with appropriate hazard symbols (biohazard, radiation, NFPA diamond, etc.). |  |  |  |  |  |  |  |
|  | The following are posted near the lab entrance: - Pink Emergency Contact Card with current contact info - Chemical Inventory - GT Emergency Procedures Sign - SDS Access Information Sign |  |  |  |  |  |  |  |
|  | Lab equipment used to manipulate biological materials is labeled with the biohazard symbol. |  |  |  |  |  |  |  |
|  | Lab freezers and refrigerators are labeled with “No Food or Drink Allowed”, “No Flammables” (if appropriate) and the biohazard symbol (if used to store biological/infectious material). |  |  |  |  |  |  |  |
| Occupational Health |  |  |  |  |  |  |  |  |
|  | All lab members that work with animals and/or biological/infectious material are enrolled in the Biosafety Occupational Health Program. |  |  |  |  |  |  |  |
|  | All lab members that are required to wear respiratory protection enroll annually into the respiratory protection program. |  |  |  |  |  |  |  |
| Engineering Controls |  |  |  |  |  |  |  |  |
| Cabinet/ Hood Certification |  |  |  |  |  |  |  |  |
|  | Chemical Fume Hoods (CFH) have been certified in the past 6 months by the Georgia Tech approved vendor and are functioning properly. The certification label is attached to the CFH. |  |  |  |  |  |  |  |
|  | CFHs that have failed certification, have not been certified within the past 6 months or are not functioning correctly (i.e., flow is not between 80-120 LFM) are tagged out of service and are not in use. |  |  |  |  |  |  |  |
|  | Biosafety Cabinets (BSC) have been certified in the past year by the Georgia Tech approved vendor and are functioning properly. The certification label is attached to the BSC. |  |  |  |  |  |  |  |
|  | BSCs that have failed certification or have not been certified within the past year are tagged out of service and are not in use. |  |  |  |  |  |  |  |
|  | All active laminar flow hoods/clean benches have been certified within the past year by the Georgia Tech approved vendor and are functioning properly. The certification label is attached and initialed by the vendor. |  |  |  |  |  |  |  |
| Cabinet/Hood Use |  |  |  |  |  |  |  |  |
|  | CFH and BSC sashes are functioning properly, set to appropriate heights, not cracked, and alarms are not muted. |  |  |  |  |  |  |  |
|  | Items are not stored on top of the BSC. |  |  |  |  |  |  |  |
|  | Bunsen burners and/or open flames are not used in the BSC. Flammable gas is not used or connected to BSC gas lines (i.e., natural gas). |  |  |  |  |  |  |  |
|  | Items stored in CFHs and BSCs do not disrupt normal use and/or airflow. Specifically, BSC grills are free from obstructions. |  |  |  |  |  |  |  |
|  | Laminar flow hoods/clean benches are not used to work with hazardous material. |  |  |  |  |  |  |  |
| Centrifuges |  |  |  |  |  |  |  |  |
|  | Centrifuges have door interlocks (mechanism to keep lid closed during operation). |  |  |  |  |  |  |  |
|  | Secondary containment (i.e., centrifuge safety caps, buckets, sealed rotors) is available and used when centrifuging infectious samples. |  |  |  |  |  |  |  |
| Emergency Equipment |  |  |  |  |  |  |  |  |
|  | A double ocular eyewash is available within 10 second access. |  |  |  |  |  |  |  |
|  | A safety shower is available within 10 second access. |  |  |  |  |  |  |  |
|  | Eyewashes and safety showers are free of obstruction for easy access during an emergency. |  |  |  |  |  |  |  |
|  | Eyewashes are tested weekly by lab members and the test is documented. NOTE: Eyewashes equipped with safety caps have them in place. |  |  |  |  |  |  |  |
|  | Safety showers are tested annually by GT Facilities and the test is documented. |  |  |  |  |  |  |  |
|  | Fire extinguishers are appropriate for the hazards in the lab, visible and accessible in the lab. |  |  |  |  |  |  |  |
|  | Fire extinguishers are visually inspected monthly by lab members. This is documented on the tag affixed to the equipment. |  |  |  |  |  |  |  |
| Personal Protective Equipment & Lab Attire |  |  |  |  |  |  |  |  |
|  | Lab coats are worn while working in the lab. |  |  |  |  |  |  |  |
|  | Reusable coats are laundered on a regular basis by an approved method. |  |  |  |  |  |  |  |
|  | Safety glasses/goggles or another type of face protection are worn at all times in the lab. |  |  |  |  |  |  |  |
|  | Gloves are worn while working in the lab and appropriate for the experiment (examples: thermal protection for -80°C freezers/liquid nitrogen, nitrile gloves for chemicals, etc.) Disposable gloves are not reused. |  |  |  |  |  |  |  |
|  | Lab members remove gloves before leaving the lab and opening doors. |  |  |  |  |  |  |  |
|  | Closed toed shoes and long pants/skirts are worn at all times in the lab. Examples of inappropriate attire include: sandals, torn jeans, and ballet flats. |  |  |  |  |  |  |  |
| Hazardous Material Storage |  |  |  |  |  |  |  |  |
|  | NFPA/Right-To-Know compliant labels are affixed to in house made containers of solutions. |  |  |  |  |  |  |  |
| Chemicals |  |  |  |  |  |  |  |  |
|  | EHSA barcode labels are present on all primary chemical containers (including gas cylinders). |  |  |  |  |  |  |  |
|  | Chemicals are segregated by hazard (i.e., acids and bases separated; acids are segregated by type: inorganic and organic). |  |  |  |  |  |  |  |
|  | Hazardous liquids are stored no higher than shoulder height. |  |  |  |  |  |  |  |
|  | Chemical containers are in good condition (i.e., no bulging, leaking, cracked caps or crystal formation). |  |  |  |  |  |  |  |
|  | Secondary containment is present for all hazardous liquids. Note: squirt bottles and working solutions (i.e., flasks beakers, etc.) are exempt from this requirement. |  |  |  |  |  |  |  |
|  | Lab members extract chemicals from one stock container until the container is empty. |  |  |  |  |  |  |  |
| Flammables |  |  |  |  |  |  |  |  |
|  | Flammables are stored in flammable safety cabinets when not in use. |  |  |  |  |  |  |  |
|  | Flammable materials are limited to 10 gallons/100 ft2 of lab space. |  |  |  |  |  |  |  |
|  | Flammables are stored in flammable safe or explosion proof refrigerators/freezers as necessary. |  |  |  |  |  |  |  |
| Compressed Gases |  |  |  |  |  |  |  |  |
|  | Gas cylinders are secured between the middle and shoulder of cylinder. NOTE: No more than two gas cylinders are secured with on restraint. |  |  |  |  |  |  |  |
|  | Gas cylinders without a regulator attached have safety caps in place. |  |  |  |  |  |  |  |
|  | Toxic or flammable gases present in the lab are compliant with the GT Dangerous Gas Safety Program. |  |  |  |  |  |  |  |
| Waste Management |  |  |  |  |  |  |  |  |
| Sharps |  |  |  |  |  |  |  |  |
|  | Unprotected sharps are not left unattended, lying out on bench tops. |  |  |  |  |  |  |  |
|  | Disposable sharps are properly disposed of in hard walled sharps container labeled with the principal investigator’s name and containers are no greater than ¾ full. |  |  |  |  |  |  |  |
|  | Needles are not bent, broken, recapped, removed from disposable syringes, or otherwise manipulated by hand before disposal. |  |  |  |  |  |  |  |
| Broken Glass |  |  |  |  |  |  |  |  |
|  | Broken glass containers with plastic liners are available and no greater than ¾ full. Lab does not use broken glass containers for the disposal of sharps, biohazard-contaminated glass, gloves, used bulbs, etc. |  |  |  |  |  |  |  |
| Chemical Waste |  |  |  |  |  |  |  |  |
|  | Chemical Waste is stored in an easily accessible location. |  |  |  |  |  |  |  |
|  | Chemical waste is properly labeled with a description of the contents, fill start date and owner’s name. NOTE: EHSA waste cards are filled out and fixed to containers ready for pick up by EHS. |  |  |  |  |  |  |  |
|  | Chemical waste is stored in compatible containers (i.e., no acid in metal, no HF in glass, etc.). |  |  |  |  |  |  |  |
|  | Chemical disposal containers are closed when not in use. |  |  |  |  |  |  |  |
|  | Liquid chemical waste is in secondary containment. |  |  |  |  |  |  |  |
| Biological Waste |  |  |  |  |  |  |  |  |
|  | Animal carcasses are double bagged in biohazard bags and refrigerated/frozen until pick-up by EHS. |  |  |  |  |  |  |  |
|  | Solid, non-sharp, biological waste is disposed of in biomedical waste boxes lined with biohazard bags (provided by EHS). These are packed for EHS pick up. |  |  |  |  |  |  |  |
|  | Liquid biological waste is labeled appropriately and disinfected prior to disposal down the drain using the chemical disinfectant and contact time indicated on the lab’s Biological Hygiene Plan. |  |  |  |  |  |  |  |
| Electrical Safety |  |  |  |  |  |  |  |  |
|  | Electrical panels are unobstructed (i.e., 3 ft of clearance in front of panels). |  |  |  |  |  |  |  |
|  | Ignition sources are segregated from flammables/combustibles. |  |  |  |  |  |  |  |
|  | Permanent equipment is plugged directly into an outlet (no extension cords). |  |  |  |  |  |  |  |
|  | Electrical cords are not frayed or damaged. |  |  |  |  |  |  |  |
| Emergency Preparedness |  |  |  |  |  |  |  |  |
|  | Lab is equipped with a spill kit. |  |  |  |  |  |  |  |
|  | Lab members have been trained on how to clean up a minor spill. |  |  |  |  |  |  |  |
|  | Lab members know how to report incidents and injuries. |  |  |  |  |  |  |  |
| Housekeeping |  |  |  |  |  |  |  |  |
|  | Lab sinks are equipped with soap and paper towels for handwashing. |  |  |  |  |  |  |  |
|  | Lab floor, bench tops and furniture are easily cleanable (i.e., can be wiped down) and can handle the anticipated loads. |  |  |  |  |  |  |  |
|  | Lab is under restricted access (i.e., doors are lockable, doors are kept closed). |  |  |  |  |  |  |  |
|  | Food/drinks/cosmetics/lotions are not present in the lab. |  |  |  |  |  |  |  |
|  | Work surfaces are disinfected with or an appropriate disinfectant after each use and are visibly clean. Bench papers are changed on a regular basis. |  |  |  |  |  |  |  |
|  | Work surfaces and aisle ways are uncluttered to allow space for safe work practices. |  |  |  |  |  |  |  |
|  | Items are not stored within 18” of the ceiling to allow for safe function of building sprinkler systems. |  |  |  |  |  |  |  |